Bytes sent to the AVR will be sent to the LCD for text display unless the LCD is in ADC or PWM display mode. Command bytes allow other functions to be set or read. After sending a command byte, one or more proceeding bytes are used to set controller configurations. ASCII values of the control bytes are DC1=17, DC2=18, DC3=19, DC4=20, ETB=23. Command bytes are as follows.

254:	The following byte is sent to the LCD as a command (R/S=0, R/W=0)

DC1:	The following byte is used to control the LCD functions

B0:	SAS	SDS	SSP	VDM	VD1	VD0	R/W	BKL

BKL		0	Backlight pin off

1	Backlight pin on

R/W		0	R/W=0

1	R/W=1. Sending any non-control character will result in the cursor position being read. The character is sent back as DC1,CHR,ETB. Sending command 254 sets R/W to zero.

VD<1:0>	00	Display characters from the UART.

01	Display PWM values on the LCD. All UART characters are ignored.

10	Display ADC values on the LCD. All UART Characters are ignored..

11	Alternate display of PWM and ADC values (~ 5 second intervals). All UART characters are ignored.

A change in VD will result in the LCD display being reset.

VDM		0	For VD modes 01-11, values are numeric.

1	For VD modes 01-11, values are displayed as an eight segment bargraph.

SAS		0	do not save settings

1	Saves all current settings to EEPROM.

R/W is set to zero

SDS is set to zero

SDS		0	do not store display strings

1	Store display strings. B1 must be sent next.

SSP		0	Startup screen cleared after ~ 5 seconds					1	Startup screen not cleared after startup

B1:	N3	N2	N1	N0	M3	M2	M1	M0

M		0000	Store the next N characters as PWM0 name.

0001	Store the next N characters as PWM1 name.

0010	Store the next N characters as PWM2 name.

0011	Store the next N characters as PWM3 name.

0100	Store the next N characters as ADC0 name.

0101	Store the next N characters as ADC1 name.

0110	Store the next N characters as ADC2 name.

0111	Store the next N characters as ADC3 name.

1000	Store the next 80 characters as startup message

1001-1111 No character storing.

Note: if M=0000-0111, N must be less than 11, or N is set to 10.

Note: SAS=1 or M=1000 will send DC1,ETB if EEPROM is already writing. Further bytes are not read as message storing bytes.

DC2:	The following five bytes are used to control the PWM/ADC functions.

B0:	MC3	MC2	MC1	MC0	AVM	SAV	SPV	KPC

MC0,1,2,3	0	USART control of PWM channel 0,1,2,3.

1	Manual control of PWM channel 0,1,2,3

KPC		0	keypad control of PWM channels off

1	Keypad control of PWM channels on, sets MC0,1,2,3 = 0

Push *,Channel,X,X,X,#

DC3,KON must be set for the keypad to function

DC1,VD!=00 for the keypad to control PWM channels

SPV		0	Do not send PWM values

1	Send PWM values to USART. Sends the following bytes: DC2,"P",PWM0,PWM1,PWM2,PWM3,ETB

SAV		0	Do not send ADC values

1	Send ADC values to USART. Sends the following bytes: DC2,"A",ADC0,ADC1,ADC2,ADC3,ETB

AVM		0	Sent ADC values are 8 bit values

1	Sent ADC values are 10 bit values (sent as two bytes, ADCnH,ADCnL, left formated)

B<4:1>:	PWM values for PWM0,1,2,3

DC3:	The following byte is used to control the Keypad

B0:	-	-	-	-	-	SK1	SK0	KON

KON		0	Keypad off

1	Keypad on

SK<1:0>		00	Do not send keypresses

01	Send last 0-32 unsent keypresses after a hash press

Sends DC3,KP0,KP1....KPN,#,ETB

10	Send all keypresses as they occur

Sends DC3,KP0,ETB

11	Do not send keypresses

DC4:	The following byte is used to control GPIO functions

BO:	IND3	IND2	IND1	IND0	INDC	GPO2	GPO1	GPO0

GPO0,1,2	0	Turns GPO0,1,2 off

1	turns GPO0,1,2 on

IND0,1,2,3	0	turns IND0,1,2,3 off if INDC=1.

1	turns IND0,1,2,3 on if INDC=1.

INDC              0	IND0,1,2,3 lines indicate ADC control of PWM channels

1	IND0,1,2,3 lines controlled via DC4 command

Command Examples:

1)	turn keypad on and request all keypresses as they occur		DC3,7

2)	Manual control of PWM0, PWM1, and PWM3			DC2,176